ZTE Wireless Modem

UserGuide for EmbeddedLinux

Note: This document is provided for information purposes only.   ZTE cannot accept any responsibility for the use of information provided in this document.

© 2026 ZTE Corporation. All rights reserved.

2026版权所有  中兴通讯股份有限公司  保留所有权利

版权声明：

本文档著作权由中兴通讯股份有限公司享有。文中涉及中兴通讯股份有限公司的专有信息，未经中兴通讯股份有限公司书面许可，任何单位和个人不得使用和泄漏该文档以及该文档包含的任何图片、表格、数据及其他信息。

本文档中的信息随着中兴通讯股份有限公司产品和技术的进步将不断更新，中兴通讯股份有限公司不再通知此类信息的更新。

1	INTRODUCTION	5

2	REQUIREMENTS	5

3	KNOWN LIMITATIONS	5

4	INSTALLING DRIVERS	6

5	SETTING UP PPP USING PPPD AND CHAT	8

6	USING DATA DEVICES WITH AUTO-INSTALL CD	11

7	TROUBLESHOOTING	13

Revision History

| REV | DATE | AUTHOR | NOTES |
| --- | --- | --- | --- |
| 1.0 | 20 May 2009 | Zhao Ming | Initial version of document. |

# INTRODUCTION

This document describes how to set up and connect to the Internet using a ZTE Wireless Modem on the EmbeddedLinux platform using open source software packages.

Please note that this document is provided to guide current Linux users to be able to useZTE Wireless Modems on any EmbeddedLinux distribution, however, the setup instructions in this document has been tested with uCLinux. For setup on other Linux distributions, you should still be able to follow the instructions after you download and install the necessary software packages outlined below. Instructions in this document are only meant to guide the user and its information may change at any time.

# REQUIREMENTS

Before you can connect to the Mobile Operator network, you will need the following:

•	Wireless modem that has been activated on a Windows machine. The modem should be associated with an account with the Mobile Operator

•	Linux installation should be running Kernel 2.6.20.x or later.

•	PPP Modem Dialing software application.

••  kernel source tree (supported versions 2.6.x)

••  compiler/binutils for kernel compilation

# KNOWN LIMITATIONS

•	System/User can not monitor card status, ie, software does not show information about signal strengths, network service type, and connectivity status.   The only

network indication available is by wireless card LED that shows whether the card is in a Mobile Operator coverage area (Solid ) or transmitting data (flashing).

•	Card updates (PRL, firmware and activation) features are not available.   This can be done on a Windows machine at least for the activation of the device.

•	Driver loading is done on the command line (can be automated by adding the commands to a startup script that is run at boot time).

# INSTALLING DRIVERS

- modify option.c which locates in \linux-source-2.6.XX\drivers\usb\serial. Be sure

of logining   with root account in terminal.

Add{USB_DEVICE(ZTE_VENDOR_ID,ZTE_PRODUCT_ID)},in option_ids[ ].

ZTE_VENDOR_ID=0x19d2

ZTE_PRODUCT_ID= vary with the ZTE modem which you use.please use `lsusb` to get it.

2.   rebuild the kernel and reboot your system.

3.	 plugin the ZTE modem.

4.  The driver can be loaded  successfully now.  Although it is not necessary to verify the result of running the command above, but if you want to be sure that the driver is now associated with your card. Plug in your card to the computer and make sure you have power then run the following on the command-line:

“dmesg|grep -i ttyUSB”

You should see something like:

usb 5-1: generic converter now attached to ttyUSB0

usb 5-1: generic converter now attached to ttyUSB1

Now the driver has been loaded and validated you need to setup the dialer.

- in general, each ZTE modem has multiports. In Linux, user can use modem port and AT port. For different ZTE modem, the device node of modem port and AT port may change.

Please contact customer manager of ZTE to get Modem interface descriptor lists

For example:

| MF636 PID:0x0017 | 0 | Diag interface | /dev/ttyUSB0 |
| --- | --- | --- | --- |
|  | 1 | AT interface | /dev/ttyUSB1 |
|  | 2 | Modem interface | /dev/ttyUSB2 |
|  | 3 | NDIS interface | /dev/ttyUSB3 |

| MF622 PID:0x0002 | 0 | NDIS interface | /dev/ttyUSB0 |
| --- | --- | --- | --- |
|  | 1 | Modem interface | /dev/ttyUSB1 |
|  | 2 | Diag interface | /dev/ttyUSB2 |
|  | 3 | AT interface | /dev/ttyUSB3 |

| MF200/210 PID:0x0016 | 0 | Diag interface | /dev/ttyUSB0 |
| --- | --- | --- | --- |
|  | 1 | AT interface | /dev/ttyUSB1 |
|  | 2 | Modem interface | /dev/ttyUSB2 |

# SETTING UP PPP USING PPPD AND CHAT

Now that we have shown you how to setup a connection using pppd and chat below is another method for those who are more comfortable with ZTE. You will still need to perform steps 2-3 so that your device can be recognized by the OS. This also assumes you have already downloaded pppd and installed on your current Linux distribution. After that you will need to run the following steps.

  - Prepare dial-scripts of pppd

the Main script of pppd, its content is following:

the first part of the Main script:Setup some parameters of pppd

#!/bin/sh

clear

/sbin/route del -net default

# These are the parameters. Change as needed.

echo "=============== This is the part 1 of the scripts. ================"

TELEPHONE=*99#	      # The telephone number for the connection

ACCOUNT=			# The account name for logon

PASSWORD=			# The password for this account

LOCAL_IP=0.0.0.0		# Local IP address if known. Dynamic = 0.0.0.0

REMOTE_IP=0.0.0.0		# Remote IP address if desired. Normally 0.0.0.0

NETMASK=255.255.255.0	# The proper netmask if needed

OPTION_FILE="zte_options"

DIAL_SCRIPTS_PATH="."

DEBUG_STAT="no"

export TELEPHONE ACCOUNT PASSWORD DIAL_SCRIPTS_PATH DEBUG_STAT

if [ "$1" = "-d" ]; then

DEBUG_STAT="yes"

$DIAL_SCRIPTS_PATH/zte_log $0 "DEBUG_STAT = " $DEBUG_STAT

fi

Note:TELEPHONE may vary with your mobile operator, *99# is valid in CHINA MOBILE

the second part of the Main script：Run pppd with some parameters

the second part of the Main script：Specify dialer-script

echo "=============== This is the part 2 of the scripts. ================ "

DIALER_SCRIPT=$DIAL_SCRIPTS_PATH/zte_ppp_dialer

$DIAL_SCRIPTS_PATH/zte_log $0 "DIALER_SCRIPT = " $DIALER_SCRIPT

if [ -f $DIALER_SCRIPT ]; then

if [ -x $DIALER_SCRIPT ]; then

$DIAL_SCRIPTS_PATH/zte_log $0 DIALER_SCRIPT "is ok!"

else

echo $DIALER_SCRIPT": permission limittion"

exit 0

fi

else

echo "The file" $DIALER_SCRIPT " don't exist in the current directory!"

exit 0

fi

echo " =============== This is the part 3 of the scripts. ================"

exec /usr/sbin/pppd $LOCAL_IP:$REMOTE_IP file $OPTION_FILE connect $DIALER_SCRIPT

the type content of OPTION_FILE is as following:

/dev/ttyUSB0

115200

noauth

noipdefault

defaultroute

usepeerdns

crtscts

lock

lcp-echo-failure 4

asyncmap 20A0000

kdebug 4

netmask 255.255.255.0

-detach

-ccp

-vj

Noted: device node may vary with you ZTE modem, so you should

Confirm whether modem node is /dev/ttyUSB0 first

- the Dialer script，this script will call by the Main script ,its content is as following:

#!/bin/sh

OPTIONS_FILE=$DIAL_SCRIPTS_PATH/"zte_chat_options"

$DIAL_SCRIPTS_PATH/zte_log $0 "OPTIONS_FILE = " $OPTIONS_FILE

$DIAL_SCRIPTS_PATH/zte_log $0 "TELEPHONE = " $TELEPHONE

#exec chat -v -s -f $OPTIONS_FILE

exec /usr/sbin/chat -v -s						\

TIMEOUT		3				\

ABORT		'\nBUSY\r'				\

ABORT		'\nNO ANSWER\r'			\

ABORT		'\nRINGING\r\n\r\nRINGING\r'	\

''		\rAT					\

OK		AT+CPIN?				\

'READY'-"AT+CPIN=\"1234\""-'OK'	AT+CPMS?	\

OK		AT+ZOPRT=5				\

OK		AT+CGDCONT=1,\"IP\",\"cmnet\"	\

''		ATDT$TELEPHONE			\

CONNECT		''				\

Noted:AT+CGDCONT  may vary with your mobile operator, cmnet is valid in CHINA MOBILE.

AT+CPIN is determined by your SIM pin code.

- the Disconnect script，its content is following:

#!/bin/sh

######################################################################

if [ "$1" = "" ]; then

DEVICE=ppp0

else

DEVICE=$1

fi

######################################################################

if [ -r /var/run/$DEVICE.pid ]; then

kill -INT `cat /var/run/$DEVICE.pid`

echo "PPP link to $DEVICE terminated."

exit 0

fi

echo "ERROR: PPP link is not active on $DEVICE"

exit 1

  - Start to dial and connect

Open a terminal window and run the Main script start to dial and connect

  - Disconnect

Open a terminal window and run the Main script start to dial and connect

The way 1:  entry “ctrl+c” at a terminal window;

The way 2 :  run the disconnect script.

# USING DATA DEVICES WITH AUTO-INSTALL CD

Working with devices that have multiple functions can sometimes be tricky and requires a few extra steps. For devices that support internal memory that is often seen as a CD drive users can often install automatically for Windows systems. This is not the case for Linux.

When the device is inserted, the auto-install CDROM will be mounted and will open up a file browser window as shown in Figure 1 using Ubuntu Live CD.   Auto-install is specific to certain multifunction devices such as the ZTE MF632 modem.   In order to be able to use the modem to establish communication, you should right-click on the CDROM drive, as highlighted in the Figure below and select —Eject“ from the context menu to unmount and eject.

Figure 1. Auto-Install CDROM of the ZTE modem is mounted and launches file browser automatically.

On distributions that do not have the capability to mount CDROM automatically, the

command line interface (also known as the Linux Shell) can be used to eject the auto-install CDROM using the following command:

sudo eject /dev/cdrom

of course , you can use usb_modeswitch which is a usb tool from internet (http://www.draisberghof.de/usb_modeswitch/) to eject CDROM.

NOTE:

CDROM device special file is allocated dynamically. It could be /dev/sr0, or /dev/sr1 …etc. Yours may vary. For my test I used /cdrom.

Assuming that —cdrom“ is the device special file associated with the auto-install CDROM that needs to be ejected. Once ejected the device should change to Modem and Storage mode.

# TROUBLESHOOTING

Dropped connection:

In the event that the connection is lost due to any reason, a manual reconnect is required. You can configure your Connection to automatically redial on disconnect.

DNS issues:

For DNS-related issues, please verify entries in /etc/resolv.conf file. Please refer to your Linux distribution‘s user manual for more information about fixing this problem.